Test Plan

Track Controller (Hardware) — Wayside Controller Module

Document Version: 1.0

Date: 2026-10-19

Prepared By: Braden (Track Controller HW owner)

Project Name: ECE1140 Train Management System — Team 3 “Track Overflow”

# Table of Contents

1. Introduction

1.1 Purpose

1.2 Scope

1.3 Definitions, Acronyms, & Abbreviations

1.4 References

1.5 Overview

2. Features to Be Tested

3. Features Not to Be Tested

4. Item Pass/Fail Criteria

5. Test Deliverables

6. Responsibilities

7. Test Items

7.1 PLC Language and Dual-Channel Execution

7.2 Wayside Database (Territory)

7.3 Controller Core: Inputs, Fail-Safe Hold, Ready, Reports

7.4 Sample PLC Programs (the Safety Logic)

7.5 Test Link and Wire Format

7.6 Track Controller Window (UI)

7.7 Test UI

7.8 System Integration (Central Harness)

8. Approvals

# 1. Introduction

## 1.1. Purpose

This plan identifies the items, features, testing tasks, personnel and risks for verifying the hardware Track Controller (wayside) module, TrackCtrlHw, against the SRS v1.0 requirements allocated to the Track Controller. It defines what is tested, how each requirement is verified (Test, Demonstration or Inspection, per SRS §4), and the criteria that decide whether the module passes.

## 1.2. Scope

Covered:

The track_ctrl_hw package (controller, wayside, PLC compiler and both scan channels, territory parser, view layer, test link and wire format).

The three Green line wayside databases (data/waysides/) and their PLC programs (data/plc/), which are the module’s safety logic.

The Track Controller window (ui/) and the separate test UI process (test_ui/), which stands in for the CTC Office, the Track Model and the clock.

System boundary. Inputs: closed blocks, maintenance switch commands and suggested speed/authority from the CTC Office; occupancy and reported switch, signal and crossing states from the Track Model; simulation time from the clock. Outputs: track circuit speed/authority, switch, signal and crossing commands to the Track Model; one report per wayside to the CTC Office. The module knows blocks, never trains.

Test levels:

Unit — automated, Python unittest, 74 cases in TrackCtrlHw/tests/ (§7.1–7.6).

Integration — two real OS processes over the test link (§7.5) now; the module stepped by the central harness with the neighbouring modules from Iteration 3 (§7.8).

System — integrated run, dispatching a train from the yard on the Hardware Track Controller’s line (Iteration 3, 2026-11-12).

Acceptance — demonstration to the customer (PAAC) at the final demo, 2026-12-10, through the integrated system or, if integration is incomplete, through the test UI.

Constraints and assumptions:

Windows 11; Python ≥ 3.10 (3.14.2 used) and PySide6 ≥ 6.11 in the project virtual environment (SRS §2.4, toolchain convention).

Fixed time step of 0.1 s (decision D006); authority is a count of blocks (D013); one shared simulation clock (D017).

The Hardware Track Controller runs every wayside of one line; the Software Track Controller runs the other line and is verified under its own plan. Test data covers Green line blocks 1–35 and 74–88 in three waysides.

There is no vital layer above the PLC program: every protection (separation, closures, switch locking, speed limits, crossings) is the program’s. The sample programs are therefore tested as the safety case, not as examples.

Risks and open issues affecting this plan:

| # | Risk / open issue | Effect on testing | Owner |
| --- | --- | --- | --- |
| R1 | REQ-FUNC-059 requires detecting broken rail, track circuit and power failures. By design the module receives a failure only as an occupied block and cannot tell which failure occurred. | Only the protective response is tested (failed block reported occupied and protected). 059.1–059.3 as worded will not pass until the SRS or the design changes. | Braden |
| R2 | REQ-FUNC-057 (know when in maintenance mode): the CTC Office sends no maintenance flag, only closures and switch commands. | Verified through CTC switch commands and closures reaching the program. Whether that satisfies 057 is undecided. | Braden / Landis |
| R3 | REQ-INTF-009 (HW/SW equivalent interfaces): PLC variable names differ (HW CTCSW_n, CTCREV_n, SPD_n_b, CMD_n_b; SW still MAINT, no speed bits). One program file does not load on both. | INT-05 is expected to fail inspection until the names are aligned. | Braden / Harry |
| R4 | The central harness (D005) does not exist yet. | §7.8 cannot run; the test UI stands in for every neighbour. | Unassigned |
| R5 | No event log was found in TrackCtrlHw (REQ-NFR-008, REQ-INTF-015). | Not tested at module level (§3). | Unassigned |
| R6 | Sample-program weaknesses: two trains on adjacent blocks hold each other (possible deadlock); authority is not truncated at an occupied block (no direction data); a train at 70 km/h (19.4 m/s) on 100 m block 26 needs ~157 m to stop at 1.2 m/s². | Not exposed by unit tests; INT-08 and INT-09 target them at system level. | Braden |
| R7 | flake8 and pylint (required by the coding standard) are not installed in the virtual environment. | Static checks cannot run until installed. | Braden |

## 1.3. Definitions, Acronyms, & Abbreviations

| Term | Definition |
| --- | --- |
| Wayside | One track controller governing a contiguous set of blocks (its territory), with its own database and PLC program. |
| Territory / wayside database | JSON file: a cut of the course track layout file plus a wayside field. |
| PLC | Programmable Logic Controller; here a user-written program of Boolean variables and operators (NOT, AND, XOR, OR). |
| Scan | One execution of the whole PLC program against the current input image; one per wayside per step. |
| Channel A / B | The two independent implementations that compile and run every statement (A: recursive descent + tree walk; B: shunting-yard + stack machine). |
| Channel fault | Channels A and B disagree on any output; latched until a new program is loaded. |
| Fail-safe hold | Every signal red, every crossing active, speed 0 and authority 0, switches where they are. |
| Ready | A wayside has a program loaded and its channels agree. The line runs only while every wayside is ready. |
| Authority | Count of blocks ahead of the train it may enter (D013). |
| Track circuit | Per-block speed and authority sent to the Track Model, which passes it to the train. |
| BlockKey / BlockRef | Track Model key (line, section, block) / CTC Office key (line, block), written Green:13. |
| Aspect | Signal indication: red, yellow, green, super green. |
| Test UI | Separate process that drives the module over the test link in place of its neighbours. |
| Central harness | Component that steps every module and maps types between them (D005). |
| CTC | Centralized Traffic Control (the CTC Office module). |
| HW / SW | Hardware / Software variant of the Track Controller. |
| SRS | Software Requirements Specification (documents/srs-filled.md). |

## 1.4. References

Software Requirements Specification v1.0, Team 3 — documents/srs-filled.md (§3.2.6 Track Controller, §4 Verification).

Module Requirements Matrix — documents/requirements-matrix.md.

Track Controller (Hardware) README — TrackCtrlHw/README.md (interface, PLC language, known gaps).

Project decisions on the truth branch: D005 central harness, D006 fixed time step, D008 interface catalog, D013 authority as a block count, D017 shared simulation clock.

UI style guide — truth/ui/style-guide.md (truth branch).

Coding Standards (Group).docx and documents/PYTHON_STYLE_GUIDE.md.

Course track data — TrackModel/green_line.json; Track Layout & Vehicle Data vF5.xlsx.

Test plan template — documents/test_plan.md.

IEEE Std 829, Standard for Software and System Test Documentation (informative).

## 1.5. Overview

Section 2 lists the features and requirements to be tested and how. Section 3 lists what is excluded and why. Section 4 defines pass, fail, suspension and resumption. Section 5 lists the artifacts produced. Section 6 assigns responsibilities. Section 7 describes each test item with its test cases, each traced to a requirement and to the automated test or manual procedure that checks it. Section 8 holds approvals.

# 2. Features to Be Tested

Method follows SRS §4. Test items refer to §7.

| Requirement | Feature | Method | Test items |
| --- | --- | --- | --- |
| REQ-FUNC-049 | Receive suggested speed and authority from the CTC Office, per block | Test | CTL-04, SMP-13, LNK-06, INT-01 |
| REQ-FUNC-050 | Load a PLC file; report every error and warning; refuse a program that does not fit the wayside | Test | PLC-04–10, CTL-07–10, UI-08, UI-09 |
| REQ-FUNC-051 | PLC language based only on Boolean variables | Inspection | PLC-12 (PLC-01–05 support) |
| REQ-FUNC-052 | Switches move by PLC program execution | Test + Demo | CTL-16, SMP-06, UI-11 |
| REQ-FUNC-053 | Signal aspects set by PLC program execution | Test + Demo | CTL-18, SMP-07, UI-11 |
| REQ-FUNC-054 | Receive train presence (occupancy) from the Track Model | Test | CTL-19, INT-04 |
| REQ-FUNC-055 | Send occupancy (and switch, signal, crossing state) to the CTC Office | Test | CTL-19, UI-06, UI-12, INT-02 |
| REQ-FUNC-056 | Activate railway crossing lights and gates | Test + Demo | CTL-17, SMP-08, UI-11 |
| REQ-FUNC-057 | Know maintenance mode (via CTC switch commands and closures; see R2) | Demo | SMP-09–12, UI-11 |
| REQ-FUNC-058, 058.1 | Safety-critical architecture: diverse two-channel execution, fail-safe hold, line stall, separation, closures, speed limits, switch locking | Test + Demo | PLC-11, CTL-11–15, CTL-21–26, SMP-01–07, SMP-11, SMP-14, INT-06, INT-08, INT-09 |
| REQ-FUNC-059 | Failure reported by the Track Model is protected as occupancy (see R1) | Test | INT-07 |
| REQ-FUNC-012 | CTC maintenance switch commands reach the wayside | Demo | SMP-09, LNK-06 |
| REQ-INTF-001.4 | GUI for the Track Controller (Hardware) | Demo | UI-01–13 |
| REQ-INTF-003, 005 | Style guide and layout convention | Inspection | UI-14 |
| REQ-INTF-004 | Consistent invalid-input error messages | Demo | PLC-08, UI-09, UI-10, TUI-04 |
| REQ-INTF-006 | Separate hardware variation exists | Inspection | INT-05 |
| REQ-INTF-009 | HW/SW equivalent external interfaces (see R3) | Inspection | CTL-01, INT-05 |
| REQ-INTF-011 | Usable at 720×450 | Demo | UI-13 |
| REQ-INTF-012 | Track layout loaded from JSON | Test | TER-01–10, CTL-05 |
| REQ-INTF-013 | Shared simulation clock | Test | UI-05, TUI-03 |
| REQ-INTF-014 | PySide6 GUI | Inspection | UI-14 |
| REQ-INTF-017 | Track circuit carries speed/authority toward the train | Test + Demo | CTL-11–15, SMP-01–05, INT-04 |
| REQ-INTF-018, 019, 020 | Channels with Track Model and CTC Office | Test + Demo | CTL-19, LNK-01, LNK-06, INT-01–04 |
| REQ-FUNC-001, 002; REQ-NFR-002 | Real time and 10×, same results | Demo | TUI-03 |
| REQ-NFR-003, 004 | Validate input; reject without changing state | Test | CTL-02, CTL-03, CTL-09, TER-07, TER-10, PLC-07, LNK-02, LNK-07, UI-09, UI-10, TUI-04 |
| REQ-NFR-005, 006 | No crashes or unhandled errors | Test | LNK-02, LNK-03, LNK-05; all manual runs |
| REQ-NFR-007 | Ready for use within 20 s of launch | Test | UI-15 |
| REQ-DSN-004 | Interaction only through defined interfaces | Inspection | CTL-01, TER-05 |

# 3. Features Not to Be Tested

| Feature | Reason for exclusion |
| --- | --- |
| Train identity, location or speed | The module knows blocks, never trains, by design (TrackCtrlHw/README.md). |
| Failure-type discrimination (REQ-FUNC-059.1–.3 as worded) | A failure reaches the module only as occupancy; it cannot tell a broken rail from a train. Open issue R1. |
| Authority truncated at an occupied block | Needs each train’s direction, which the module does not receive. Known gap; the sample programs hold the train instead. |
| Moving Block Overlay | Out of scope (decision D001). |
| Software Track Controller | Separate implementation, owner and test plan. Only interface equivalence (INT-05) is checked here. |
| Behaviour of the CTC Office, Track Model, Train Model and Train Controllers | Covered by those modules’ plans. They are replaced by the test UI or exercised only at their boundary with this module. |
| Event log (REQ-NFR-008, REQ-INTF-015) | Not found in the module; expected to be a shared utility (REQ-DSN-005). Risk R5. |
| Standalone executable (REQ-DSN-001, 002) | Packaging is a system-level task, verified in the system test plan. |
| PLC programs other than the three samples | No vital layer exists; the safety of any other program is its author’s responsibility. Only compile-time checks (§7.1) apply to them. |

# 4. Item Pass/Fail Criteria

| Criteria | Description |
| --- | --- |
| Pass (test case) | The observed result equals the expected result in §7. For automated cases: the unittest case reports OK. |
| Pass (requirement) | Every test case traced to the requirement in §2 passes. |
| Pass (module) | All 74 automated cases pass (python -m unittest discover -s tests -t . from TrackCtrlHw/), all manual procedures in §7.6–7.7 pass, and no critical or major defect is open. |
| Fail | Any automated case fails or errors, or any manual procedure differs from its expected result. |
| Critical defect | Any failure of a safety case (fail-safe hold, channel fault, line stall, separation, closure, speed cap, switch locking, crossing activation) or a crash. Blocks the milestone until fixed and retested. |
| Suspension | Testing stops if the environment cannot run the suite (PySide6 missing, test link cannot start a second process) or a critical defect makes further results meaningless. |
| Resumption | After the cause is fixed: rerun the full automated suite plus every manual procedure affected by the change. |
| Regression | The full automated suite runs on every commit to the module and before every merge to development. |

# 5. Test Deliverables

This test plan (TrackCtrlHw/docs/Track_Controller_HW_Test_Plan.docx).

Automated test suite: TrackCtrlHw/tests/ — test_plc.py, test_territory.py, test_controller.py, test_link.py, test_wire_and_views.py, with support.py and link_server.py.

Test fixtures: three wayside databases (data/waysides/), three sample PLC programs (data/plc/), and error/warning example programs (data/plc/examples/).

Test UI (test_ui/) and its link (track_ctrl_hw/link.py, wire.py, harness.py), kept working through the final demo.

Screenshots from ui/capture.py for UI layout checks.

Test results report: automated run log and manual procedure checklist with results, per milestone (WP3 “test plan results”).

Defect reports in JIRA using the team’s Defect Report form.

# 6. Responsibilities

| Role | Responsibility | Owner |
| --- | --- | --- |
| Module owner / test author | Writes and maintains this plan and the automated tests; runs unit and link tests; performs manual procedures; fixes defects. | Braden |
| SW Track Controller owner | Reviews interface equivalence (INT-05, R3) and the shared PLC language. | Harry |
| CTC Office owner | Confirms the CTC edge mappings (INT-01–03) and the maintenance-mode question (R2). | Landis |
| Track Model owner | Confirms occupancy, reported state and failure behaviour at the boundary (INT-04, INT-07). | [Unassigned] |
| Central harness owner | Provides the harness and runs §7.8. | [Unassigned] |
| Reviewer / approver | Reviews and approves this plan (§8). | [Unassigned] |

# 7. Test Items

Test case IDs are TCHW-<item>-<nn>; the TCHW- prefix is omitted in the tables. “Check” names the automated test (file::Class.method, file names without test_ prefix) or the manual method.

## 7.1. PLC Language and Dual-Channel Execution

Item Description: track_ctrl_hw/plc.py compiles a Boolean PLC program (VAR_IN / VAR_OUT declarations with ranges, := assignments over NOT, AND, XOR, OR, constants 0/1, parentheses, // comments) and scans it on two independently written channels.

Version/Build: feature/TrackCtrl-Showcase-Feedback @ 305ff58 (2026-10-16)

Test Approach: Automated unit tests. Differential testing of the two channels: 3000 random expressions (fixed seed 1140, depth ≤ 5) × 16 input combinations, each channel compared with an independent reference evaluator. Grammar inspected for Boolean-only content.

Test files / fixtures: tests/test_plc.py (11 cases).

| ID | Test case (condition / stimulus) | Expected result | Req. | Check |
| --- | --- | --- | --- | --- |
| PLC-01 | Evaluate A OR B AND C, NOT A AND B, A XOR B AND C, A OR B XOR C | Binding NOT > AND > XOR > OR; channels agree | FUNC-051 | plc::Grammar.test_precedence_not_and_xor_or |
| PLC-02 | Lower-case keywords, parentheses, constants 1 and 0 | X=1, Y=1, Z=0 for A=B=0 | FUNC-051 | plc::Grammar.test_parentheses_constants_and_case |
| PLC-03 | A temporary reassigned between statements | Statements run once, in order: X=0, Y=1 | FUNC-051 | plc::Grammar.test_assignments_run_in_order_once |
| PLC-04 | Ranges OCC_1 .. OCC_3, SW_A..C, AUTH_9..11 | Expanded in order | FUNC-050 | plc::Grammar.test_ranges |
| PLC-05 | Read of an undeclared, unset name | Reads 0 | FUNC-051 | plc::Grammar.test_undefined_read_is_zero |
| PLC-06 | Compile a 1-statement program | Checksum XXXX-XXXX (hex); Boolean and statement counts correct | FUNC-050 | plc::Grammar.test_checksum_and_counts |
| PLC-07 | 12 malformed programs: assign to an input, incomplete or unbalanced expression, non-bit constant, missing operator, keyword as value, = for :=, unknown character, bad name, backwards range, empty expression | Each yields an error; program not accepted | FUNC-050, NFR-003 | plc::Diagnostics.test_errors |
| PLC-08 | Errors on lines 1 and 3 of one file | Both reported, with line numbers | FUNC-050, INTF-004 | plc::Diagnostics.test_every_bad_line_is_reported |
| PLC-09 | Duplicate declaration, read before set, output reassigned, output never assigned | Each a warning, not an error | FUNC-050 | plc::Diagnostics.test_warnings |
| PLC-10 | Comments and blank lines | No diagnostics | FUNC-050 | plc::Diagnostics.test_comments_and_blank_lines |
| PLC-11 | 3000 random expressions × 16 inputs | Channel A = channel B = reference in every case | FUNC-058 | plc::Channels.test_random_expressions_match_a_reference |
| PLC-12 | Review the grammar in plc.py | Only Boolean variables, 0/1 and Boolean operators; no integers, comparisons or timers | FUNC-051 | Inspection |

## 7.2. Wayside Database (Territory)

Item Description: track_ctrl_hw/territory.py parses a wayside database: block number, section, length, speed limit, switch string and railway crossing. It derives each switch’s point, normal and reverse ends, places a signal at every switch, and refuses malformed files.

Version/Build: feature/TrackCtrl-Showcase-Feedback @ 305ff58 (2026-10-16)

Test Approach: Automated unit tests, including a byte-for-byte comparison of the three sample databases with the course layout file, and refusal cases for malformed databases and switch strings.

Test files / fixtures: tests/test_territory.py (10 cases); data/waysides/*.json; TrackModel/green_line.json.

| ID | Test case (condition / stimulus) | Expected result | Req. | Check |
| --- | --- | --- | --- | --- |
| TER-01 | Load green_wayside_1.json | Green, wayside 1, 20 blocks; SW-12 point 13, normal 12, reverse 1; crossing E-19; 45 km/h = 12.5 m/s | INTF-012 | territory::SampleWaysides.test_wayside_1 |
| TER-02 | Load green_wayside_2.json | SW-28 reverse leg to block 150, outside the territory | INTF-012 | territory::SampleWaysides.test_wayside_2_leg_leaves_territory |
| TER-03 | Load green_wayside_3.json | SW-76 (77/76/101) and SW-85 (85/86/100) | INTF-012 | territory::SampleWaysides.test_wayside_3_two_switches |
| TER-04 | Compare samples with green_line.json | Every block identical | INTF-012 | territory::SampleWaysides.test_samples_match_the_course_layout_file |
| TER-05 | Grade, elevation, station, beacon fields present | Ignored | INTF-012, DSN-004 | territory::Parsing.test_ignores_fields_of_other_modules |
| TER-06 | Numeric wayside and block IDs | Stored as strings | INTF-012 | territory::Parsing.test_ids_are_strings |
| TER-07 | No wayside, no blocks, duplicate block, zero or Boolean length, non-Boolean crossing, not an object | TerritoryError; nothing loaded | NFR-003, NFR-004 | territory::Parsing.test_refusals |
| TER-08 | Switch strings as in the course files | Correct point, normal and reverse | INTF-012 | territory::SwitchStrings.test_forms_in_the_course_files |
| TER-09 | 57-yard, Yard-63 | Reverse end is the yard | INTF-012 | territory::SwitchStrings.test_yard_switches |
| TER-10 | 8 malformed switch strings (no shared block, loop, three connections, point outside the wayside, …) | TerritoryError | NFR-003 | territory::SwitchStrings.test_refusals |

## 7.3. Controller Core: Inputs, Fail-Safe Hold, Ready, Reports

Item Description: track_ctrl_hw/controller.py (HwTrackController, implementing the TrackController Protocol in interface.py) and wayside.py. step(dt, inputs) validates all inputs before any state changes, scans every wayside once, and returns Track Model commands and one CTC report per wayside. Outputs are held fail-safe while no program runs, a new program has not scanned, the channels disagree, or any wayside on the line is not ready.

Version/Build: feature/TrackCtrl-Showcase-Feedback @ 305ff58 (2026-10-16)

Test Approach: Automated unit tests driving step() with constructed inputs. Fault injection by patching channel B to invert one output. Snapshot comparison before and after rejected input. Minimal one-line programs show the wayside adds nothing above the program.

Test files / fixtures: tests/test_controller.py: Contract, Loading, FailSafe, NothingAboveTheProgram, Report, Ready (26 cases); tests/support.py.

| ID | Test case (condition / stimulus) | Expected result | Req. | Check |
| --- | --- | --- | --- | --- |
| CTL-01 | Inspect HwTrackController | Implements step, snapshot, load_territory, load_program, reset | DSN-004, INTF-009 | controller::Contract.test_satisfies_the_protocol |
| CTL-02 | dt = 0, −0.1, ∞, NaN, True, "0.1" | InvalidTimeStepError | NFR-003 | controller::Contract.test_rejects_bad_time_steps |
| CTL-03 | Negative or NaN time; speed −1, 5.5 or 32 m/s; switch position "sideways"; BlockKey where BlockRef is due | InvalidInputError; snapshot unchanged | NFR-003, NFR-004 | controller::Contract.test_rejects_bad_inputs_before_any_change |
| CTL-04 | Suggestion for a Red line block | Ignored; no track circuit output | FUNC-049 | controller::Contract.test_ignores_blocks_it_does_not_govern |
| CTL-05 | Load a Red database beside a Green one | TerritoryError | INTF-012 | controller::Loading.test_one_line_only |
| CTL-06 | Second wayside claims blocks of the first | TerritoryError | INTF-012 | controller::Loading.test_a_block_belongs_to_one_wayside |
| CTL-07 | Reload the same database | Program kept | FUNC-050 | controller::Loading.test_reload_keeps_a_program_that_fits |
| CTL-08 | Reload a smaller database the program no longer fits | Program unloaded; one notice | FUNC-050 | controller::Loading.test_reload_unloads_a_program_that_does_not_fit |
| CTL-09 | Program naming an absent block or switch; syntax error; unknown wayside | PlcError | FUNC-050, NFR-004 | controller::Loading.test_program_must_fit_the_wayside |
| CTL-10 | Load the three sample programs | No warnings | FUNC-050 | controller::Loading.test_samples_compile_cleanly |
| CTL-11 | No program; 10 m/s, 4 blocks suggested on occupied block 3 | Track circuit (0, 0); signal red; crossing active; reason “No PLC program is loaded.” | FUNC-058 | controller::FailSafe.test_no_program_holds_everything |
| CTL-12 | Load a new program | Fail-safe until its first scan, then released | FUNC-058 | controller::FailSafe.test_new_program_holds_until_its_first_scan |
| CTL-13 | Channel B inverts AUTH_3 | (0, 0); fault names AUTH_3; stays latched on healthy scans; cleared only by a new program | FUNC-058 | controller::FailSafe.test_channel_disagreement_is_a_latched_channel_fault |
| CTL-14 | AUTH_3 := 1 on closed block 3 | Authority passes, speed 0: the wayside overrides nothing | FUNC-058 | controller::NothingAboveTheProgram.test_a_closed_block_is_the_programs_to_honour |
| CTL-15 | CMD copies SPD; 30 m/s on a 12 m/s block | 30 m/s sent as the program says | FUNC-058 | controller::NothingAboveTheProgram.test_the_speed_limit_is_the_programs_to_honour |
| CTL-16 | SW_12 := 1, blocks 12 and 13 occupied | Switch 12 commanded reverse | FUNC-052 | controller::NothingAboveTheProgram.test_a_switch_moves_under_a_train_if_the_program_says |
| CTL-17 | XING_19 := 0, block 19 occupied | Crossing inactive | FUNC-056 | controller::NothingAboveTheProgram.test_a_crossing_follows_the_program |
| CTL-18 | Lit bits: none; G+SG; R+SG; Y+G; SG | red; green; red; yellow; super_green | FUNC-053 | controller::NothingAboveTheProgram.test_signal_outputs_decode_in_a_fixed_order |
| CTL-19 | Waysides 1–3; t = 3600 s; blocks 4, 22 occupied; signal 12 lit yellow | Reports for waysides 1, 2, 3 in order; sent_at 3600; 20 blocks in report 1; occupancy, switch, signal, crossing correct, None where the block has none | FUNC-054, FUNC-055, INTF-018, INTF-020 | controller::Report.test_one_report_per_wayside_keyed_by_block |
| CTL-20 | reset() | Ticks 0; no report; programs kept; switch normal | — | controller::Report.test_reset_forgets_scans_but_keeps_programs |
| CTL-21 | Nothing loaded | Not ready (“No wayside database is loaded.”); no reports | FUNC-058 | controller::Ready.test_nothing_loaded_is_not_ready |
| CTL-22 | Database, no program | Report ready = false with reason | FUNC-058 | controller::Ready.test_a_wayside_without_a_program_is_not_ready |
| CTL-23 | Wayside 1 ready, wayside 2 without a program; then load wayside 2’s program | Whole line held fail-safe; released once every wayside is ready | FUNC-058 | controller::Ready.test_the_line_stalls_until_every_wayside_is_ready |
| CTL-24 | Channel fault | Report not ready; reason names AUTH_3 | FUNC-058 | controller::Ready.test_a_channel_fault_is_not_ready |
| CTL-25 | New program on a ready wayside; reset() | Stays ready | FUNC-058 | controller::Ready.test_ready_survives_a_new_program_and_a_reset |
| CTL-26 | Database the program no longer fits | Not ready | FUNC-058 | controller::Ready.test_a_database_the_program_no_longer_fits_loses_ready |

## 7.4. Sample PLC Programs (the Safety Logic)

Item Description: data/plc/green_wayside_{1,2,3}.plc (205, 158 and 175 lines). With no vital layer, these programs carry every protection: separation, closures, switch locking and correspondence, crossings, speed limits, and obedience to CTC maintenance switch commands.

Version/Build: feature/TrackCtrl-Showcase-Feedback @ 305ff58 (2026-10-16)

Test Approach: Automated scenario tests on the loaded samples, including an exhaustive speed-cap check (every speed 0–31 m/s on every block of all three waysides). Manual review of each program against the protection list in the README.

Test files / fixtures: tests/test_controller.py: SamplePrograms, Maintenance (13 cases).

| ID | Test case (condition / stimulus) | Expected result | Req. | Check |
| --- | --- | --- | --- | --- |
| SMP-01 | Blocks 3, 8, 9 occupied; suggestions on 3 and 8 | Block 3 keeps (12, 4); block 8, beside occupied 9, held (0, 0) | FUNC-058.1 | controller::SamplePrograms.test_separation |
| SMP-02 | 30 m/s suggested on a 45 km/h block | 12 m/s sent (highest whole m/s ≤ 12.5) | FUNC-058 | controller::SamplePrograms.test_speed_clamped_to_the_whole_limit |
| SMP-03 | Every speed 0–31 on every block of waysides 1–3 | min(speed, limit) sent; authority unchanged | FUNC-058 | controller::SamplePrograms.test_every_speed_capped_on_every_block |
| SMP-04 | Block 4 occupied; suggestion on block 3 | (0, 0): held authority carries no speed | FUNC-058.1 | controller::SamplePrograms.test_held_authority_sends_no_speed |
| SMP-05 | Blocks 3, 13, 29, 77 closed with suggestions | (0, 0) | FUNC-058 | controller::SamplePrograms.test_closed_block_sends_zero |
| SMP-06 | Train on 1; then on 12; then clear | Reverse; held reverse (locked); normal | FUNC-052 | controller::SamplePrograms.test_switch_moves_by_program_then_locks_under_a_train |
| SMP-07 | Switch 12 commanded normal, reports reverse | Signal 12 red; authority 0 on blocks 13 and 1; block 5 unaffected | FUNC-053, FUNC-058 | controller::SamplePrograms.test_disagreeing_switch_reddens_its_signal_and_route |
| SMP-08 | Train on 18, 19, 20; on 17; none | Crossing active; inactive; inactive | FUNC-056 | controller::SamplePrograms.test_crossing_active_for_a_train_at_or_beside_it |
| SMP-09 | CTC commands normal while a train on 1 would reverse it; then reverse | Program obeys the CTC Office; request shown | FUNC-012, FUNC-057 | controller::Maintenance.test_the_program_obeys_ctc_switch_commands |
| SMP-10 | Program ignoring CTCSW_12 | Wayside does not override it | FUNC-057 | controller::Maintenance.test_commands_reach_the_program_as_inputs |
| SMP-11 | CTC reverse while block 13 occupied | Stays normal (locked) | FUNC-058 | controller::Maintenance.test_ctc_command_still_locked_under_a_train |
| SMP-12 | CTC commands end | Program resumes control | FUNC-057 | controller::Maintenance.test_program_resumes_when_commands_end |
| SMP-13 | Close Green:3 | (0, 0); closure recorded on the wayside | FUNC-049 | controller::Maintenance.test_closures_are_named_by_line_and_block |
| SMP-14 | Review each sample program | Implements every protection listed in the README | FUNC-058 | Inspection |

## 7.5. Test Link and Wire Format

Item Description: track_ctrl_hw/link.py (QLocalServer/QLocalSocket, newline-delimited JSON, one client at a time) and wire.py (JSON form of every boundary type, with the CTC Office’s part in its own shapes).

Version/Build: feature/TrackCtrl-Showcase-Feedback @ 305ff58 (2026-10-16)

Test Approach: Integration tests across two real OS processes (tests/link_server.py started as a subprocess); JSON round-trip and malformed-message unit tests.

Test files / fixtures: tests/test_link.py (5 cases); tests/test_wire_and_views.py: Wire (2 cases).

| ID | Test case (condition / stimulus) | Expected result | Req. | Check |
| --- | --- | --- | --- | --- |
| LNK-01 | Same 5-step input sequence via the link and locally | Identical outputs | INTF-018, 019, 020 | link::Link.test_remote_outputs_equal_local_ones |
| LNK-02 | dt 0; negative speed, over the link | Typed InvalidTimeStepError / InvalidInputError; link keeps working | NFR-004, NFR-006 | link::Link.test_rejections_come_back_typed |
| LNK-03 | Second client connects | Refused as busy; first client unaffected | NFR-006 | link::Link.test_one_test_ui_at_a_time |
| LNK-04 | Reset over the link | Outputs cleared; switches normal | — | link::Link.test_reset |
| LNK-05 | Non-JSON, non-object, incomplete lines, then a good request | Error replies; good request answered | NFR-005, 006 | link::Link.test_bad_lines_get_errors_and_do_not_block_good_ones |
| LNK-06 | Inputs, outputs, territories through JSON | Equal after the round trip; CTC part as {line, block_id} and {line, switch_id, position} | INTF-019, 020, FUNC-012 | wire_and_views::Wire.test_round_trips_through_json |
| LNK-07 | 12 malformed messages | WireFormatError | NFR-003 | wire_and_views::Wire.test_rejects_malformed_messages |

## 7.6. Track Controller Window (UI)

Item Description: The window that hosts the module (ui/main.py, QML) and its Qt-free view layer: display.py (imperial units), schematic.py (territory diagram), views.py (table rows), state.py and rows_model.py.

Version/Build: feature/TrackCtrl-Showcase-Feedback @ 305ff58 (2026-10-16)

Test Approach: Automated tests on the Qt-free view layer. Manual demonstration on the running window driven by the test UI. Headless capture (ui/capture.py) for layout at minimum size. Inspection against the UI style guide.

Test files / fixtures: tests/test_wire_and_views.py: Schematic, Display, Views (7 cases); launch.py; ui/capture.py; data/plc/examples/.

| ID | Test case (condition / stimulus) | Expected result | Req. | Check |
| --- | --- | --- | --- | --- |
| UI-01 | Schematic of wayside 1 | 20 columns, one row; points at 12; reverse leg loops to block 1; crossing at 18.5 | INTF-001.4 | wire_and_views::Schematic.test_loop_back_to_a_block_of_the_wayside |
| UI-02 | Schematic of wayside 3 | Legs leaving the territory drawn as stubs “TO 101”, “TO 100” | INTF-001.4 | wire_and_views::Schematic.test_legs_that_leave_the_wayside_are_stubs |
| UI-03 | Y branch (Blue 5-6 / 5-11) | Branch on its own row | INTF-001.4 | wire_and_views::Schematic.test_a_y_branch_gets_its_own_row |
| UI-04 | 12 m/s; 100 m; 1 and 0 blocks | 27 mph; 328 ft; “1 block”, “0 blocks” | INTF-001.4 | wire_and_views::Display.test_conversions_use_the_convention_factors |
| UI-05 | Clock: none; 14:32:07.9; one day + 5 s | --:--:--; 14:32:07; 00:00:05 | INTF-013 | wire_and_views::Display.test_clock |
| UI-06 | Occupied 4, 9; closed 7; suggestion on 4; signal 12 green | Block, office, switch, maintenance and report rows show the right states and imperial values | INTF-001.4, FUNC-055 | wire_and_views::Views.test_rows_show_display_units_and_states |
| UI-07 | Summary of wayside 3 | “Sections M–O · Blocks 74–88” | INTF-001.4 | wire_and_views::Views.test_summary |
| UI-08 | Run python TrackCtrlHw/launch.py; Load database for each of the three files; New PLC for each | All three waysides in the picker; header Not ready until every wayside has a program, then Ready | FUNC-050, INTF-001.4 | Demo |
| UI-09 | New PLC with data/plc/examples/errors.plc, then warnings.plc | Every diagnostic listed with its line number; errors refuse the program, warnings do not; same message format as other errors | FUNC-050, INTF-004, NFR-004 | Demo |
| UI-10 | Load an invalid database file | Refused with a message; loaded waysides unchanged | NFR-004, INTF-004 | Demo |
| UI-11 | From the test UI: occupancy, closure, suggestion, CTC switch command, reported switch/signal | Schematic, block table, track circuit column, switches, signals & crossings and maintenance banner update in imperial units | FUNC-049, 052–054, 056, 057 | Demo |
| UI-12 | Open PLC program details and Last report | Checksum, channel A/B agreement, fault and hold shown; last report matches the test UI’s CTC outputs | FUNC-055, 058 | Demo |
| UI-13 | Resize to 720×450; capture.py --demo | All panels readable; 16:10 aspect kept | INTF-011 | Demo |
| UI-14 | Compare with truth/ui/style-guide.md | Tokens, shared components and layout convention followed; PySide6/QML used | INTF-003, 005, 014 | Inspection |
| UI-15 | Time from launch to usable window | ≤ 20 s | NFR-007 | Test (stopwatch) |

## 7.7. Test UI

Item Description: test_ui/ with track_ctrl_hw/harness.py: a separate process that stands in for the CTC Office, the Track Model and the clock (1× or 10×) and drives the module only through step(). It is supported through the final demonstration.

Version/Build: feature/TrackCtrl-Showcase-Feedback @ 305ff58 (2026-10-16)

Test Approach: Manual demonstration. The link it relies on is verified automatically in §7.5.

Test files / fixtures: test_ui/main.py; launch.py.

| ID | Test case (condition / stimulus) | Expected result | Req. | Check |
| --- | --- | --- | --- | --- |
| TUI-01 | Start the test UI before, then after, the module window | Connects whenever the window listens; picks up loaded waysides | INTF-001.4 | Demo |
| TUI-02 | Start a second test UI | Refused as busy | NFR-006 | Demo (see LNK-03) |
| TUI-03 | Run the clock at 1×, then 10×; Advance; Reset module | Ticks advance at the chosen rate; outputs update every tick; same inputs give the same outputs at both rates (fixed 0.1 s step) | FUNC-001, 002, NFR-002, INTF-013 | Demo |
| TUI-04 | Enter speed 32 m/s, negative authority, malformed block | Rejected with a message; nothing sent; module state unchanged | NFR-003, 004, INTF-004 | Demo |

## 7.8. System Integration (Central Harness)

Item Description: HwTrackController stepped by the central harness (D005) together with the CTC Office, the Track Model and the Software Track Controller on the other line. Planned for Iteration 3 (2026-11-12); not executable until the harness exists (R4).

Version/Build: feature/TrackCtrl-Showcase-Feedback @ 305ff58 (2026-10-16)

Test Approach: Integration and system tests in the harness. Each cross-module edge mapping listed in the README is tested explicitly. Failure injection from the Track Model UI.

Test files / fixtures: To be written with the harness.

| ID | Test case (condition / stimulus) | Expected result | Req. | Check |
| --- | --- | --- | --- | --- |
| INT-01 | CTC suggestion for a train on Green:13 | Mapped to BlockRef; track circuit on block 13 | FUNC-049, INTF-019 | Integration test |
| INT-02 | Wayside reports flattened for the CTC Office | CTC map shows occupancy, switch and crossing state | FUNC-055, INTF-020 | Integration test |
| INT-03 | One wayside not ready | CTC Office shows the line not ready | FUNC-058 | Integration test |
| INT-04 | Track Model occupancy and reported states in; commands out | Commands reach the Track Model; track circuit reaches the train | INTF-017, 018 | Integration test |
| INT-05 | Compare HW and SW interfaces and PLC names | Same Protocol and types; one program loads on both (fails today, R3) | INTF-006, 009 | Inspection |
| INT-06 | Dispatch a train from the yard to a station | Train runs with authority and speed within limits; no collision | FUNC-058.1 | System demo |
| INT-07 | Inject broken rail, circuit and power failures | Block reported occupied; trains held from it | FUNC-059 | System test |
| INT-08 | Two trains on adjacent blocks | Neither deadlocks the line (R6) | FUNC-058.1 | System test |
| INT-09 | Train at 19.4 m/s approaching a held block 26 | Stops within its authority (R6) | FUNC-058.1 | System test |

# 8. Approvals

| Name | Role | Date |
| --- | --- | --- |
| Braden | Track Controller (HW) owner, author |  |
| Harry | Track Controller (SW) owner, reviewer |  |
| [Name] | [Role] |  |